T3000-DTF-SY

DTF printer instruction manual

○ Thank you for purchasing this product

○ Please read this manual carefully before use ○ Please keep this book properly

Contents

1.Product Introduction	3

2. Model specification	3

3. Product show and rack installation	4

3.1 Product show	4

3.2 Rack installation	5

4. Software installation and setup	6

4.1 Computer configuration requirements	6

4.2 Installation of printing software	6

4.2.1 Printing software installation	6

4.2.2 IP address set	7

4.2.3 Connect Gigabit Network Cable	7

4.3 FlexiPRINT software installation (RIP)	8

4.3.1 Activation of RIP software	8

4.3.2 Software installation	10

4.3.3 Software setup and use	12

4.4 MainTop software installation (RIP)	17

4.4.1 Software installation	17

4.4.2 Software setup and use	18

4.5 Machine power detection	21

4.6 Ink tank installation	22

4.7 Print head installation position and cable connection	23

4.7.1 Print Head Cable to Adapter Board Connection	25

4.8 Debug before printing	27

4.8.1 Print Nozzle Test	27

4.8.2 Print Head Vertical Alignment	28

4.8.3 Step calibration	28

4.8.4 Print Head Horizontal Spacing Calibration	30

4.8.5 Print Head Vertical Spacing Calibration	31

4.8.6 Bidirectional print calibration	32

4.9 Printer setup	33

4.10 Maintenance	33

4.11 Enter advanced setup	34

5.Printer Panel Operation Instructions	35

5.1 Panel introduction	35

5.2 Button function description	36

5.3 Menu function introduction	36

6.Precautions for printing equipment	38

1.Product Introduction

A3 printer, small size, easy to operate, offset hollow heat transfer printing machine, one-key printing hollow pattern, white and color at the same time, no need for lettering, hollowing, engraving, no waste discharge, no pollution, one-stop printing of heat transfer products, convenient Fast, is a major innovation in the history of digital printing, quality assurance.

- Model specification

| Model Type | T3000-DTF |
| --- | --- |
| Print head | XP600 |
| Resolution | 1440DPI |
| Speed | 8pass: 3 m2/h 6pass: 4 m2/h |
| Color control | ICC color profile with adjust function |
| Print Width | 300 mm |
| Ink | Pigment |
| Operation Environment | Temperature 18°C~28°C, Humidity 50%-70% |
| Power | 50HZ/60HZ 220V 10A |
| System | Windows7 or Above |
| Applicable materials | Cotton, Nylon, Chemical fiber, leather, etc. |
| Image format | Tiff, Jpeg, Eps, Pdf，Png |
| Printer size | Machine size：967 X 430 X 417mm(LXWXH) |
|  | Packing size：1340 X 490 X 560mm(LXWXH) |
| Printer weight | Net weight： 58 KG Gross weight：120 KG |
| Powder machine size | Machine size：500 X 678 X 470mm(LXWXH) |
|  | Packing size：715 X 560 X 575mm(LXWXH) |
| Powder machine weight | Net weight： 37 KG Gross weight：40 KG |

- Product show and rack installation

  - Product show

| - 01 Carriage Ink Stack Assembly | - 02 Control Panel | - 03 Acrylic Cover |
| --- | --- | --- |
| - 04 Pressure Roller Rod | - 05 Ink Tank Holder | - 06 Ink Tank |
| - 07 Discharge bracket left | - 08 Discharge bracket right | 09 Steel pipe |
| - 10 Discharge tray | - 11 Adjustment control part |  |

- 12 Drying and heating part

- 13 Dusting and shaking powder part

- 14 Powder tank section

- 15 Control Panel Section

- 16 Material winding section

3.2 Rack installation

Step 1: Open the box of the machine, remove the buffer foam block and protective film, and take out all accessories to check whether the host and accessories are damaged and whether the appearance is worn. Note: random accessories, check carefully to prevent accessories from being lost!

Step 2: Install the unloading bracket, install the left and right unloading brackets separately, and fix them with screws. as the picture shows:

Right

Left

Step 3: Install the steel pipe and the discharge tray, and install the left and right discharge trays separately. as the picture shows:

Step 4: Install the printing material, lift the right pressure lever, put in the printing material, and then press down the right pressure measuring lever. The printing material is installed in the middle of the printing plane, and the two sides are pressed with paper pressing sheets. as the picture shows:

4. Software installation and setup

4.1 Computer configuration requirements

- Computer configuration requirements:

- 1. Computer system: Windows7 Ultimate 64-bit computer system.

- 2. CPU configuration: use i5 or above i5 CPU.

- 3. Memory stick: 8G or above.

- 4. Network card and network cable requirements: Gigabit or above network card must be used, and the network cable needs to be super Category 6 or above to support dry-mega communication network cable.

- 5. Hard disk configuration: It is recommended to use a 120G solid state hard disk as the system disk, and then add a mechanical hard disk as a secondary disk as needed.

4.2 Installation of printing software

4.2.1 Printing software installation

Open CD or USB to find the compressed package, and extract the file.

- Open the folder and find the software icon ，Double-click to run the software directly.

4.2.2 IP address set

Printer IP：192.12.0.25

Local IP：192.12.0.188

4.2.3 Connect Gigabit Network Cable

We will ship the machine with a Gigabit Ethernet cable. Then we connect one end of the Gigabit Ethernet cable to the network port of the printer motherboard, and the other end to the network port of the computer.

Insert one end into the board

Insert the other end into the computer

4.3 FlexiPRINT software installation (RIP)

4.3.1 Activation of RIP software

Instructions for activation of RIP software:

- Start the browser and enter the activation website in the browser https://www.saicloud.com press enter and enter the activation code box on the software box label after entering the web page.

- Select I am a new user, enter the email address and create an account, a confirmation email will be sent to the mailbox. Enter the mailbox, click the link in the email, enter the account password and other registration information to complete the account registration.

Click Activate Now, click the Download License button to download the license file

4.3.2 Software installation

- Put the installation CD into the CD-ROM drive or open the USB disk, open the Autorun executable file in the directory, select the installation language, accept the software license agreement and set the software installation directory.

- When installing to the pop-up license dialog box,If the currently installed computer cannot be connected to the Internet, directly import the previously downloaded license file.

- If the current computer can access the Internet, you can enter the activation code to complete the licensing process or click "Get Authorization from Website" to import the license file.

- Finally, after the completion dialog box pops up, the software installation is completed.

4.3.3 Software setup and use

1.Driver file replacement

First open the software installation location disk folder, find the path of the driver folder: \Program Files (x86)\SAi\FlexiPRINT 19 Titan Edition\OutputDrivers, copy and paste all the new driver files into the "OutputDrivers" folder (copy and replace ).

2.Add device

The computer is plugged into the dongle USB.

Run icon  ，Open the software, click "Settings" in the menu bar-"Add Device"

The printer brand：Titan

The printer model：TT-SY-XP600-A3 。

How to connect to a computer:TCP/IP

Communication TCP/IP address: 127.0.0.1

After confirming, click Done.

- 3.Manually add ICC files

- Click "Setup" in the menu bar-"Default Job Properties..."

Select "Color Management"-"Output Configuration File"-"Add", select the corresponding ICC configuration file, and then add the ICC.

- 4. Set the default job properties

- Click "Setup" in the menu bar-"Default Job Properties..."

Some general default job properties are set as follows:

【Layout】:The media size and width are set according to the width of the printing film, and the height is the default value.

【Mirror】: Enable the mirror function

[Workflow]: After outputting, set to Hold.

[Color Management]: Choose a different preset mode on the preset, and the color correction will be changed to a different curve accordingly.

White spot color choke function: choke---spot 1---Type: choke, distance: 0.1mm (modified according to different needs)

[Printer Options]: Spot color ink options: select all spot colors.

[Color adjustment]:Channel: spot color 1. (Adjust white ink output ink volume) Spot color input: 100%; output: X% (X is adjusted between 1-100). Finally, after setting all the parameters, click "OK" to save the settings.

- 5. Save a preset name and set it as the default job attribute.

- Customers can adjust the parameters according to their own needs and choose the corresponding ICC profile.

- Set different settings on the preset, which can be saved as different names.

Finally, after all parameters are set, click "OK" to save the settings.

Add assignment RIP picture

Open the "Job" icon on the main interface of the software, and directly add the print image file that needs to be processed (TIF format file is generally used)

Nest Jobs: When there are multiple pictures that need to be printed together, after adding multiple pictures, select the picture that needs to be typeset together, and then right-click the mouse to select "Nest Jobs".

Job attributes:

1. Job size, you can scale the picture size according to your needs; (to check manual nesting)

2. Position, the distance setting from picture to page edge and header; (to check manual nesting)

3. Copies, you can copy multiple pictures and typeset at the same time according to your needs;

4. Spacing, when multiple pictures are typeset, set the horizontal and vertical spacing of pictures;

5. Manual nesting, you can manually adjust the position of the picture after checking;

6. Mirror, image output and printing; (to cancel manual nesting)

7. Rotate ,choose the direction of the image rotation according to your needs. (To check manual nesting)

4.4 MainTop software installation (RIP)

4.4.1 Software installation

- Put the installation CD into the CD-ROM drive or open the installation package USB disk,Find the "SETUP.EXE" running file in the installation package, and double-click to run the installation program.

- Select the language, click "OK", continue to select the software installation path directory, click "OK".

- Wait for the installation to complete, click "Finish" after the installation is complete, and restart the computer.

- Insert the dongle U disk into the computer and run the MianTop software.

4.4.2 Software setup and use

- 1. Printer settings, find the printer driver file, and install the MainTop printer.

- 2.Set the printer port

- Find the installation path: \MainTop\MON, run the "mt_mon.exe" program, and open the "MainTop RIP Management System" interface.

- Set the IP address to: 127.0.0.1

- The RAW port number is: 9100

- 3. Printer properties settings

- Select the corresponding settings for print resolution, print orientation,

- paper size, and paper type according to your needs.

- The print port : RAW_127.0.0.1

- The white ink settings are as follows

- Custom White Invaginate: 40

- Channel Number: 1

- White Mode: Spot Color

- White Channel num: cover

- White Density: Normal

- After all settings are made, confirm to save.

- 4. Open a new

- Click the menu "New ..." or the shortcut key "Ctrl+N". Set the paper canvas size according to your needs.

- 5. Add a job

- Click the menu "Import Picture" or the shortcut key "Ctrl+I". Open the file to be added, do not adjust or adjust the paper as required, and click OK.

- 6. Typesetting

- ①Import Picture ②Print ③Copy ④Paste

- ⑤Alignment and Distributing ⑥Transform and Copy ⑦Metric Panel

- The tools and icons that are commonly used in typesetting are marked.

- 7. Send and print

- Print settings, modify parameters according to requirements, and finally click "Auto-set Paper" to cut the paper canvas.

- Click "Print" to send the print file to the printer for direct printing.

4.5 Machine power detection

1. Power safety

Note: For the safety of people and machines, be sure to connect the ground wire!

Use a digital multimeter to measure whether the power supply voltage is normal, and the range of live and neutral measurement values: 210-240VAC (the multimeter is set to 750V AC gear).

- 2. Power on the machine, check the reset of the trolley

- First loosen the screws that fix the trolley for transportation. When the machine is not powered on, pull the trolley back and forth with your hand left and right to see if it is smooth.

- Then find the power cord in the machine accessories, one end is connected to the 220V power supply, and the other end is plugged into the power socket of the machine.

- Rotate and release the emergency stop button, turn on the power switch of the machine, and wait for the machine to reset.

- 3. Software online test

- After the machine is powered on, the trolley will first self-check that it is normal. Double-click to open the control software, click "Online" to print the software and the machine is online normally.

- On the operation panel of the printer, control the left and right of the trolley and whether it is running normally before and after stepping. Click Clean to confirm whether the ink stack absorbs ink and the ink scraping height is normal.

- After confirming that the actions are normal, first use the RIP software to create a file, send the print file to simulate printing, and let the machine simulate printing to check whether the printing is normal.

4.6 Ink tank installation

1. Install and fix the screws on the main ink tank bracket, then put the ink tanks in order, and connect the corresponding color ink tanks and the corresponding marked ink tubes.

2. The ink tube of white ink is connected, the ink tube marked "IN" is connected to the upper IN end of the white ink tank, and the ink tube marked "OUT" is connected to the lower OUT end of the white ink tank. Connect the power cord of the white ink stirring motor.

3. Install the battery in the liquid level alarm buzzer of the white ink tank, insert the connection terminal, and finally add the corresponding color ink to the corresponding ink tank.

4. The waste ink pipe is connected to the upper end of the waste ink bucket. When the waste ink is full, the valve of the ink bucket can be manually opened to discharge the waste ink.

Stirring motor

OUT

Waste ink tank

Waste ink outlet

Waste ink tube

Ink Tank Air Filter

K

Y

M

C

W

IN

4.7 Print head installation position and cable connection

1. Cable installation, the print head cable is one 29P cable on the left and right. (Note: Be careful when plugging in to avoid damaging the printhead or cables!)

After the cable is plugged in, the cable interface is sealed with tape to prevent foreign objects or ink from entering the cable interface.

Left

Right

2. The color print head is fixed

Put the print head to the left and back, close to the bottom plate, and install and fix it. (Note: do not fix the print head mounting screws too hard)

- 3. Nozzle and damper connection

- Set the ink sequence: first connect the ink tube to the corresponding ink tank,

- Then use a syringe to pump ink from the ink tank to the damper,

- It is better to draw about 20ML of ink for each ink path!

- The ink level in the damper of each ink path should be at the same height,

- Basically about 2/3 of the damper capacity!

- Then connect the damper to the nozzle in sequence,

- From left to right are: K Y C M C M .

4. White print head fixed

Put the print head to the left and back, close to the bottom plate, and install and fix it. (Note: do not fix the print head mounting screws too hard)

5. Print head and damper connection

Set the ink sequence: first connect the ink tube to the corresponding ink tank,

Then use a syringe to pump ink from the ink tank to the damper,

It is better to draw about 20ML of ink for each ink path!

The ink level in the damper of each ink path should be at the same height,

Basically about 2/3 of the damper capacity!

Then connect the damper to the print head in sequence,

From left to right: WWWWWWW.

4.7.1 Print Head Cable to Adapter Board Connection

The connection order of the cables corresponds to the label of the adapter board and the print head in turn (Note: plugging and unplugging the print head cable must be operated when the power is turned off, remember that the print head cable is left and right, do not reverse it, it will cause damage the print head!)

H2-W (white)

H1-C (color)

Left

Right

Left

Right

Note: The cable socket is 30P, the cable is 29P, and the cable is aligned and installed on the right side.

(The leftmost one of the socket is vacant)

Make sure the print head cable is connected correctly, turn it on, and observe whether the "HV" and "3V3" lights on the head panel are on. If the light is not on, it is abnormal. Power off immediately and check.

Connection Diagram                   Head board led light

Right

Left

4.8 Debug before printing

4.8.1 Print Nozzle Test

Power on the machine, enter the printing software interface, and connect the software to the machine. (Note: After the machine self-test is completed, the ink stack should seal the nozzle)

After the connection is successful, the lower left corner of the software will display "Connect OK".

Open the “Test” /“More” Maintenance window of the software, click "Pump" to fill the ink to confirm that the ink is flowing out of the waste ink tube, and click "Stop" to cancel the ink filling.

Finally, click "Clean" to clean the wiper.

Make sure the platform is free of obstacles, load the material, click "Nozzle test" to test the nozzle, print the nozzle test, and confirm that the nozzle is in good condition.

If the nozzle is not in good condition, repeat the above operations, and repeat "Pump" and "Clean" several times until the nozzle is in the best condition.

4.8.2 Print Head Vertical Alignment

Enter the software option "Maintenance" interface, and the test option prints "Vertical test".

If there is a deviation, loosen the fixing screw of the print head and adjust the position of the print head. You can "push the print head up to the left". Then Print Adjust again, reach to form a line.

For vertical calibration, the vertical direction of the test chart required to be printed is a straight line. The vertical calibration chart below shows that the print head has been tilted and needs to be calibrated.

4.8.3 Step calibration

Step calibration.

Click the "Step Align" button, print the step calibration wireframe, measure the distance B (unit: mm) of the step calibration wireframe height, fill in the "Distance", and click "Calculate".

Then click "Tune" to print the step calibration line. The calibration plot has a value of -2 and the lines overlap completely.

So enter the value -2 in the corresponding input box and click Calculate.

Print "Tune" again and confirm that the lines at value 0 overlap best. Click the "Save" .

4.8.4 Print Head Horizontal Spacing Calibration

To calibrate the horizontal distance between the print heads, print "Two head align".

The calibration plot is best at a value of 0, where the lines overlap completely.

If the lines at "0" do not overlap, find the overlapped line corresponding to the value below, and increase or decrease the corresponding value in the input box.

Print the calibration chart again and verify that the lines at value 0 overlap best. Click the "Save" button.

4.8.5 Print Head Vertical Spacing Calibration

Calibrate the vertical spacing between nozzles. Prints "Vertical Distance".

The calibration plot is best at a value of 0, where the lines overlap completely.

If the lines at "0" do not overlap, find the overlapped line corresponding to the value below, and increase or decrease the corresponding value in the input box.

4.8.6 Bidirectional print calibration

Finally, do a bidirectional print calibration and print the "Hidi. Align" .

The calibration plot is best at a value of 0, where the lines overlap completely.

If the lines at "0" do not overlap, find the overlapped line corresponding to the value below, and increase or decrease the corresponding value based on the original value in the input box.

Print the calibration chart again and verify that the lines at value 0 overlap best. Click the "Save" button.

Finally, switch to a different speed to calibrate the bidirectional print value and save it.

4.9 Printer setup

Dirc

One-way: The trolley is only printed once from leaving the starting point to returning to the starting point.

Bidi.: The car is printed twice from leaving the starting point to returning to the starting point.

Soft

Off: The feathering effect is turned off;

Low: low feathering effect;

Medium: feathering effect;

High: high feathering effect

(Note: The output of feathering is reduced, but the picture effect is good)

Speed

Speed 1: Low speed

Speed 2: Standard

Speed 3: High Speed

Head

Right: Only the right nozzle is selected for printing;

Left: Only the left nozzle is selected for printing;

Double heads: Both the left and right print heads are selected to print

Strip (function: the ink is ejected from the nozzle before printing the picture, so that the nozzle is in a better state)

Off: Color bar function is off;

Left: Only the left color bar function is enabled;

Right: Only the right color bar function is enabled

Both sides: enable the function of color bars on the

left and right sides at the same time;

Width: the width of the color bar;

Span: The distance between the color bar

and the printed picture.                         Left                Right           Both sides

4.10 Maintenance

Nozzle test

Print Nozzle test

Test page

Load PRN format file for printing

More...

Maintenance

Clean: perform automatic cleaning

Head capped: close the print head and the ink cartridge

Pump: Ink loading

Strong Pump: Manual Cleaning

Flush: the print head flashes

Stop: stop everything

Test

After printing the calibration strip, if you need to adjust, you need to manually adjust the nozzle position

Import and export files

Import profile: import the modified parameters into the tool

Export profile: save the modified parameters to the computer

PC keypad

Print origin: After moving the trolley left and right, click this button to locate the printing origin.

Cart. pos.: After moving the cart left and right, click this button to locate the ink station origin.

Spit pos. positioning: After moving the trolley left and right, click this button to locate the flash injection position.

Blade pos.: After moving the trolley left and right, click this button to locate the scraper position.

Home: Click this button, the trolley will return to the original position.

Left, Right: Click to move the cart left and right.

Fwd, Back: tap to feed and back paper.

Stop car, Stop paper: Click to stop the trolley movement or the paper feed movement.

4.11 Enter advanced setup

Advanced align (password: 16753)

Voltage adjust

Increase or decrease the voltage according to actual needs (voltage is limited between 450 and 600)

Voltage type

According to the ink properties, environmental factors, paper material and other factors, select the best ink output waveform, generally use S waveform, and use U or A waveform in special cases

Color

Ink order: fill in by color from left to right in the test strip

Advanced align

Step Align

When debugging the step value corresponding to the pass, click the button corresponding to the number of passes, print the step calibration box, measure the actual distance (unit: mm), fill in the "measurement distance", and click "Calculate".

Left/Right align

Click the "Left/Right Align" button, select a specific nozzle, print a calibration bar, find the line group that matches each group, and fill in the value into the corresponding box

5.Printer Panel Operation Instructions

5.1 Panel introduction

1. Standby interface after power on: You can see a basic printing parameter of the device, and you can see whether the connection is successful in the computer logo in the lower right corner.

2. Panel operation buttons: Up (up), Down (down), Left (left), Right (right),Return (exit/return), Enter (confirm)

5.2 Button function description

| 编号 | Name | Function |
| --- | --- | --- |
| 1 | Up | Press and hold directly on the standby interface, and the printing platform will move backwards (receiving direction.) |
| 2 | Down | Press and hold directly on the standby interface, and the printing platform will move forward (feeding direction). |
| 3 | Left | Press and hold directly on the standby interface, and the trolley moves to the left to the starting point of printing. Moving to the left can set the starting position of the trolley for printing. |
| 4 | Right | Press and hold directly on the standby interface to enter the equipment maintenance menu: nozzle test, automatic cleaning, manual inking, nozzle scraping, ink flushing, head, and you can also move the trolley to the right. |
| 5 | Return | After entering the menu, you can return to the previous menu, and you can cancel the print job when printing is paused |
| 6 | Enter | Press this key to enter the menu, you can set printer parameters, functions, etc., when setting parameters, press this key to confirm |

5.3 Menu function introduction

- 1. Equipment maintenance:

- 1.1. Nozzle test: It is a function used to check the status of the nozzle to see if the nozzle has broken needles and no ink.

- 1.2. Automatic cleaning: The function of cleaning the heads is automatically performed. If the nozzle test finds that the nozzle state is not good, it can be automatically cleaned. After the cleaning is completed, use the nozzle detection to print again and observe the nozzle status.

- 1.3. Manual inking: It is generally used when ink is applied to a new machine and the ink nozzle is cleaned.

- 1.4. Squeegee nozzle: After the nozzle is cleaned automatically or manually with ink, there will be ink accumulation and residue on the surface of the nozzle. At this time, the nozzle can be used to clean the residual ink on the surface of the nozzle.

- 1.5. Ink flushing: the nozzle performs flashing, this function is used when the nozzle is slightly empty or the needle is slightly broken

- 1.6. Head: When the print head finishes printing and enters standby, the head needs to be moisturised to prevent the print head from drying and clogging the print head.

- 2. Auxiliary function:

- 2.1. Flash jetting back to the warehouse: The flash jetting settings after returning to the warehouse, including flash jetting interval, flash jetting volume, etc., are set according to actual needs.

- 2.2. Heating setting: Set the heating mode of the device (front heating/post heating).

- 2.3. Ink stirring: Set the ink stirring time, set how long the interval is, and then how long to stir, and what is the stirring level.

- 2.4. Empty paper alarm: control the switch of the empty paper sensor.

- 2.5. Suction setting: Set the suction mode and intensity.

- 2.6. Printing flash jet: The flash jet settings during the printing process, including flash jet interval, flash jet volume, flash jet position, white jump wait, etc., are set according to actual needs. Waste ink extraction setting: Set the time and amount of waste ink extraction.

- 2.7. Heating module: The setting of the heating module on the device.

- 2.8. Ink flushing cycle: set the flushing time, the ink extraction time, and the ink extraction level.

- 3. Setting menu

- 3.1. LCD contrast: adjust the contrast of the LCD screen, you can adjust 1-16 gears after entering.

- 3.2. Standby automatic maintenance: Set the interval for automatic cleaning and maintenance of the equipment in standby.

- 3.3. Device Address: Enter the address of the device.

- 3.4. Ink station settings:

- - Ink station origin: Set the origin of the ink station. After entering the setting, use the left and right buttons to adjust the position of the trolley. It needs to be adjusted to the position where the ink stack is aligned with the nozzle.

- -Maintain the flashing position: Set the flashing position of the print head. After entering the setting, use the left and right buttons on the OP panel to adjust to the upper position of the ink stack.

- - Squeegee position: Set the position of the squeegee, and the position of the squeegee needs to be set based on the scraped nozzle. Every squeegee has all the nozzles scraped.

- - Blade height: Set the height of the blade when scraping the nozzle.

- -Flash jet height: Set the height of the nozzle from the ink stack position during flash jet.

- -Head height: set the position of the ink stack head in standby.

- 3.5. Print Settings:

- - Orientation: Select the printing direction, there are unidirectional printing and bidirectional printing.

- - Speed: Select the print speed, there are standard and high speed.

- - Feathering: Select printing feathering, there are four options for printing feathering, feathering off, feathering low, feathering medium, and feathering high.

- 4. Device Information: Display the registration information of the device

- 5. Factory settings: restore factory settings to use

- 6. Language: Change the display language of the device (Chinese/English)

6.Precautions for printing equipment

1. After starting the machine, the nozzle must be cleaned.

2. Print a test strip once for each switch on and off.

3. Close the software before shutting down and check whether the car is reset.

4. The machine must be maintained once a week.

When the machine is not in use, turn off the switch of the ink pump tube clamp, and turn it on again when the machine is in use to prevent ink loss

Do not touch the hot surface of the printer platform

Notes for users using Epson lift ink stack:

Do not manually push the trolley to move before the machine is turned on, it will damage the ink stack structure and scratch the print head.

The ink stack is a precision mechanism, and should not be deformed by external force.

The lubricating level of the ink stack screw and positioning tip will decrease after a period of application, and the user should add lubricating oil regularly. (Butter, recommend 3 and check once)

Once the front, back, left and right positioning of the ink stack is adjusted, the position cannot be adjusted at will or the origin of the machine can be changed.

Precautions for machine repair and maintenance:

The ambient temperature was kept at room temperature (15-30°C).

Switch on and off the machine once each test strip, if there is a disconnection, deal with it in time.

The machine must be maintained once a week, cleaning dust, refueling the rails, etc.

The ink sac must be replaced every three months to avoid clogging of the nozzle.

Do not plug or unplug any line on the machine with power on, otherwise it will cause damage to the main board and the nozzle.

The machine must be connected to the ground wire, otherwise the ink will fly, or the individual ink dots of the print head will be damaged.

Do not use a knife to manually cut the paper on the machine, which will cause the machine to scratch and affect the appearance.

Tips:

The nozzle must pay attention to sealing and moisturizing.

The machinery and equipment should be regularly maintained and maintained.

In case of holidays, the machinery and equipment should be well maintained.

If you have any questions, you can contact the relevant personnel at any time. And I wish your company a prosperous development and abundant financial resources.